NIS2PME Cybersecurity Policy Repository for SMEs

DOCUMENT TEMPLATE

Aligned with: NIS2 (Portuguese Cybersecurity Legal Framework and CNCS Regulation No. 756/2026) | QNRCS

NOTICE OF USE The documents in the NIS2PME repository are reference templates developed to help small and medium-sized enterprises implement cybersecurity practices. NIS2 is a European directive (Directive (EU) 2022/2555). In Portugal, it applies through the Portuguese Cybersecurity Legal Framework (RJC), approved by Decree-Law No. 125/2025 of 4 December, and through Regulation No. 756/2026 of the National Cybersecurity Centre (CNCS), which sets out the minimum measures of the National Cybersecurity Reference Framework (QNRCS) and the MyCiber electronic platform. These templates do not constitute legal advice and do not replace an analysis of the specific legal framework of each organisation. Before adopting these documents, the organisation should adapt them to its own circumstances, validate their suitability with those responsible for IT and, where necessary, consult a specialised legal adviser. All fields marked with [square brackets] must be replaced with the organisation's actual details.

NIS2PME  |  Internal Use  |  Confidential

[ORGANISATION NAME]

[Address | Tax ID (NIF) | Contact]

NETWORK SECURITY POLICY

Política de Segurança da Rede

Version 1.0   |   [Date]   |   Classification: Confidential - Internal Use

Aligned with: NIS2 (Portuguese Cybersecurity Legal Framework and CNCS Regulation No. 756/2026) | QNRCS | ISO/IEC 27002:2022 | CIS Controls v8.1

# 1. Document Control

Document information

| Author | [name] |
| --- | --- |
| Document owner | [name / role] |
| Creation date | [dd/mm/yyyy] |
| Last review | [dd/mm/yyyy] by [name] |
| Next review | [dd/mm/yyyy] |
| Version | 1.0 |
| Classification | Confidential - Internal Use |

Version history

| Version | Date | Approved by | Description |
| --- | --- | --- | --- |
| 1.0 | [dd/mm/yyyy] | [name] | Initial version |

# 2. Introduction and Objectives

Computer network security is the first line of defence of [ORGANISATION] against external attacks and internal threats. A well-designed and properly protected network prevents cybercriminals from mapping the infrastructure, disrupting communications, unlawfully collecting data or accessing critical applications and devices.

This Network Security Policy sets out the minimum requirements for the design, configuration, management and monitoring of the network infrastructure of [ORGANISATION], ensuring the confidentiality, integrity and availability of the systems and data that depend on it.

This policy should be read together with the Identity, Authentication and Access Control Policy, the Vulnerability and Patch Management Policy, the Asset Management Policy and the Cybersecurity Incident Response Plan (CIRP).

Regulatory reference: RJC, Article 26(1) (ensuring the security of network and information systems), and Article 27(1)(d) (security in the acquisition, development and maintenance of network and information systems), (g) (encryption) and (i) (multi-factor authentication and secured communications). The minimum measures per level (Basic, Substantial and High) are set out in Annex III to Regulation No. 756/2026. This policy can be used as it stands at the Basic level; rules that are only mandatory from a higher level say so in the text itself, and section 15 lists the QNRCS controls covered.

# 3. Scope

This policy applies to the entire network infrastructure of [ORGANISATION], including:

- Local area networks (LAN) and Internet connections.

- Wireless networks (Wi-Fi) managed by the organisation.

- VPN connections for remote access.

- Network devices: firewalls, routers, switches, Wi-Fi access points.

- Cloud services connected to the corporate network.

- All staff, service providers and third parties who use or manage the organisation's network.

# 4. Physical Security of the Network Infrastructure

Physical security is the foundation of network security. Without adequate physical protection, all logical controls can be bypassed.

## 4.1 Location and protection of equipment

- Critical network components (firewalls, routers, switches) must be housed in dedicated racks, in server rooms or cabinets with restricted physical access.

- Access to these spaces and equipment must be limited to the IT Manager and to expressly authorised personnel.

- Data and power cables must be organised, protected from physical damage and properly labelled.

- Network equipment must be protected against environmental threats: temperature, humidity, fire and power surges.

- Decommissioned equipment must be removed from the infrastructure and its configuration erased before disposal or reuse.

## 4.2 Physical network ports

- Unused switch ports must be disabled in the switch configuration.

- In publicly accessible areas or areas with weak physical control, network access control techniques must be used (e.g. MAC address filtering or quarantine VLANs).

- Installing or modifying network equipment requires authorisation and must be carried out by the IT Manager or under their direct supervision.

# 5. Network Architecture and Segmentation

The network of [ORGANISATION] must be organised into separate trust zones (VLANs). Segmentation limits the spread of malware, reduces the attack surface and makes traffic monitoring easier. The fundamental principle is: 'everything blocked by default, only what is strictly necessary allowed'.

## 5.1 Recommended network zones

The following table defines the recommended network zones. [ORGANISATION] adapts them to its own circumstances, but always keeps, as a minimum, the separation between users (CORP), servers (SERV, if there are on-premises servers), guests (GUEST) and IoT devices and printers (IOT). The management zone (MGMT) is recommended; if the equipment does not support it, wherever this policy refers to the MGMT zone this means “from identified administration workstations”.

| Zone / VLAN | Name | Systems covered | Protection |
| --- | --- | --- | --- |
| CORP | Users | Staff desktops and laptops. Controlled access to the Internet and to servers according to business need. | Medium |
| SERV | Servers | File, e-mail, application and database servers. No direct Internet access. | Very high |
| MGMT | Management | Management interfaces of firewalls, switches and other network equipment. Access restricted to the IT Manager. No Internet access. | Critical |
| GUEST | Guests | Visitor devices and unmanaged personal equipment. Internet access only, with no communication with internal networks. | Low |
| IOT | IoT / Printers | Printers, IP cameras, sensors and other devices that are not directly managed. Isolated from user and server networks. | High |

Regulatory reference: RJC, Article 27(1)(d); QNRCS PR.RI-1 (Basic level: firewalls installed and with up-to-date rules on all networks, segmentation and segregation of the network of critical systems, wireless access points securely configured, managed and monitored). ISO/IEC 27002:2022, control 8.22 (segregation of networks). Segmentation limits the impact of an incident and prevents lateral movement by attackers.

## 5.2 Mandatory segregation rules

- End-user devices (workstations, laptops) must be separated from servers.

- Unmanaged devices (visitors, personal devices) must only access a guest network (GUEST) with no connection to internal networks.

- IoT devices (printers, IP cameras, sensors) must be isolated in a dedicated VLAN.

- The management interfaces of network equipment must only be accessible from the management VLAN (MGMT).

- If there are development or test environments, they must be separated from the production environment and must not use real data (mandatory from the Substantial level; QNRCS PR.RI-2).

# 6. Firewall and Traffic Control

VLANs must be separated by firewalls with explicit rules. The principle is 'everything blocked by default, only what is necessary allowed'. No connection between different trust zones may be allowed without a rule documented and approved by the IT Manager.

## 6.1 Traffic between zones

The following table defines the traffic rules between network zones. Any exception requires a documented and approved firewall rule:

| Source | Destination | Rule |
| --- | --- | --- |
| Internet | SERV (servers) | BLOCKED by default. Allowed only if the server has a published service (e.g. e-mail, VPN). Requires a documented rule. |
| Internet | CORP / MGMT / IOT | ALWAYS BLOCKED. No exceptions. |
| CORP (users) | Internet | ALLOWED for browsing and work services. Non-work traffic may be limited (e.g. video streaming). |
| CORP (users) | SERV (servers) | CONTROLLED. Only the ports and services needed for work. A documented rule is mandatory. |
| CORP (users) | MGMT | BLOCKED. Only the IT Manager accesses the management VLAN. |
| GUEST (guests) | Internet | ALLOWED. Internet access only. |
| GUEST (guests) | CORP / SERV / MGMT | ALWAYS BLOCKED. No exceptions. |
| IOT | Internet | CONTROLLED. Only what the devices need to operate. Unnecessary ports blocked. |
| IOT | CORP / SERV / MGMT | BLOCKED. IoT devices must not communicate with user or server networks. |
| MGMT | All zones | ALLOWED for management. Access restricted to the IT Manager using dedicated credentials. |

## 6.2 Firewall configuration and maintenance

- Firewall rules must be documented with: source zone, destination zone, protocol, port, business justification and approval date.

- Overly permissive rules (e.g. 'any to any') are prohibited without explicit approval and a defined expiry date.

- Firewall rules must be reviewed at least every six months. Outdated or unjustified rules must be removed. The outcome of each review (date, rules changed or removed, person responsible) is recorded in the review log in section 14.

- Management access to the firewall must be restricted to the management VLAN (MGMT), with mandatory multi-factor authentication (MFA).

- The default administration credentials of all network equipment must be changed at installation.

- Browsing filtering (web or DNS) is in place that blocks known malicious destinations, on the firewall or through a contracted service (QNRCS PR.SP-3, Basic level, which also requires e-mail filtering).

Regulatory reference: ISO/IEC 27002:2022, controls 8.20 (networks security) and 8.21 (security of network services). CIS Controls v8.1, control 12 (network infrastructure management). QNRCS PR.SP-1 (Basic level: secure baseline configuration, recorded in section 9.1) and PR.RI-1.

# 7. Remote Access and VPN

Remote access to internal systems of [ORGANISATION] from untrusted networks (home, customer premises, public places) is only made through the organisation's VPN or a zero trust access solution (ZTNA, Zero Trust Network Access), always with multi-factor authentication (MFA). The VPN encrypts traffic and ensures that only authenticated users reach the internal network. Services deliberately published on the Internet (e.g. cloud e-mail, website) are not remote access, but the accounts that use them follow the Identity, Authentication and Access Control Policy.

| Requirement | Specification |
| --- | --- |
| Protocol | IPsec/IKEv2, OpenVPN or WireGuard (or an equivalent ZTNA solution). PPTP and L2TP without IPsec are prohibited. |
| Encryption | Current algorithms: AES (preferably AES-GCM) or ChaCha20-Poly1305. Do not use DES, 3DES or RC4. |
| Authentication | MFA mandatory: user credentials + second factor (TOTP or push notification). |
| Split tunnelling | Disabled by default. All traffic goes through the VPN tunnel when connected. Exceptions require documented approval. |
| Session timeout | Idle VPN session disconnected after [60 minutes]. Maximum session of [8 hours] without re-authentication. |
| Authorised devices | Only devices managed by [ORGANISATION] or previously approved by the IT Manager, entered in the remote access register (section 7.2). |
| Access logging | All VPN access (connections, disconnections, failures) is logged and kept for at least [12 months]. |
| Third-party access | External providers with VPN access use dedicated accounts, with access restricted to the minimum necessary and with automatic expiry. From the Substantial level, remote maintenance of critical systems is logged and reviewed, and access is revoked as soon as the maintenance ends (QNRCS PR.GA-7). |

PROHIBITED: The use of obsolete and insecure VPN protocols is prohibited: PPTP and L2TP without IPsec. Direct access to internal systems from the Internet without VPN or ZTNA is prohibited, except for a documented, time-limited exception approved by the Cybersecurity Officer.

## 7.1 Good practice for VPN users

- Do not share VPN credentials with third parties.

- In public places, do not access critical systems without an active VPN, even on connections that seem secure.

- Report immediately to the IT Manager any failure or unusual behaviour of the VPN connection.

Regulatory reference: RJC, Article 27(1)(g), (h) and (i); QNRCS PR.GA-3 (Basic level: MFA for remote access and privileged accounts), PR.GA-7 (Basic level: list of users and devices authorised for remote access and security controls on remote connections) and PR.SD-2 (Basic level: cryptographic mechanisms for data in transit). ISO/IEC 27002:2022, control 6.7 (remote working).

## 7.2 Register of authorised remote access

Only those on this list have remote access. The IT Manager keeps it up to date and reviews it every six months, together with the access review; access that is no longer needed is removed (QNRCS PR.GA-7, Basic level).

| User or provider | Authorised device | Systems accessed | Approved by, date and validity |
| --- | --- | --- | --- |
| [name] | [laptop no. / identifier] | [e.g. file server, ERP] | [name], [dd/mm/yyyy], until [dd/mm/yyyy] |
| [name] | [identifier] | [systems] | [name], [dd/mm/yyyy], until [dd/mm/yyyy] |
| [IT provider] | [provider's equipment] | [e.g. firewall, servers] | [name], [dd/mm/yyyy], until [dd/mm/yyyy] |

# 8. Wireless Network Security (Wi-Fi)

Wi-Fi networks are an attack vector in their own right, because the signal extends beyond the premises. [ORGANISATION] uses WPA3 or WPA2-Enterprise (802.1X) whenever the equipment allows it (QNRCS PR.RI-1, Basic level: wireless access points securely configured, managed and monitored).

## 8.1 Authentication and encryption standards

| Protocol / Standard | Status | Notes |
| --- | --- | --- |
| WPA3 (Personal or Enterprise) | RECOMMENDED | Most secure current standard. Preferred on the internal network and on all new equipment. |
| WPA2-Enterprise + AES (802.1X) | RECOMMENDED | Individual authentication per user or device (RADIUS server). Alternative for the internal network when WPA3 is not supported. |
| WPA2-Personal + AES (PSK) | ACCEPTABLE | Acceptable on the guest network, with a strong password changed regularly. On the internal network only temporarily, if the equipment supports neither WPA3 nor WPA2-Enterprise: long random password, changed whenever someone who knew it leaves, and replacement of the equipment planned. |
| WEP, WPA with TKIP or open network | PROHIBITED | Obsolete, broken or unauthenticated protocols. Never use them, not even on temporary guest networks. |

## 8.2 Configuration and management of Wi-Fi networks

- There must be separate SSIDs for corporate users, guests and IoT devices.

- The guest network must have Internet access only, with no communication whatsoever with internal networks. Isolation between clients (client isolation) must be enabled.

- Access points (APs) are managed by the organisation. The six-monthly review looks for unauthorised access points (rogue APs), for example in the Wi-Fi controller console or with a Wi-Fi analyser app; any found are switched off.

- The firmware of the APs must be updated regularly, in line with the Vulnerability and Patch Management Policy.

- The guest network password (PSK) must be changed at least quarterly.

PROHIBITED: Creating personal Wi-Fi networks (hotspots, tethering) from organisation equipment connected to the corporate network is prohibited without the express authorisation of the IT Manager. Installing unauthorised Wi-Fi access points is also prohibited.

# 9. Network Management and Administration

## 9.1 Network documentation

A well-documented network is essential for a fast incident response and for effective management. The IT Manager must keep the following documents up to date:

- Network diagram: includes zones, VLANs, firewalls, main devices and Internet connections. A printed copy must be kept in a secure place. Mandatory from the Substantial level, with the segmentation of critical systems and the internal communication flows (QNRCS PR.RI-1 and ID.GA-3); at the High level, also the external flows (e.g. VPN, Internet connections), approved and kept up to date.

- Network device inventory: model, firmware version, location, IP addressing and owner; it is part of the asset inventory in the Asset Management Policy (QNRCS ID.GA-1).

- IP addressing table: ranges per VLAN, static reservations and DNS servers.

- Firewall rule register: source, destination, service, justification and approval date.

- Secure baseline configuration: for each type of network equipment, the reference configuration applied (default credentials changed, unnecessary services and ports disabled, management only over SSH or HTTPS, event logging and time synchronisation enabled), with the date of the last change (QNRCS PR.SP-1, Basic level).

- List of Internet-facing assets: firewall, VPN, servers and published services, with the details required by Regulation No. 756/2026, Article 32. It is kept with the inventory in the Asset Management Policy, reported to the CNCS through MyCiber and updated annually.

All these documents must be updated whenever there are significant changes and reviewed at least annually.

## 9.2 Secure administration of network devices

- Access to management interfaces (web, CLI) must be restricted to the management VLAN (MGMT) and to duly authorised administrative users.

- Management interfaces must not be directly accessible from the Internet. Remote administrative access must be made through the VPN.

- Unencrypted management protocols (Telnet, HTTP) are prohibited; use SSH and HTTPS.

- Multi-factor authentication (MFA) is mandatory for access to the management interfaces of network equipment, whenever the equipment supports it; if it does not, access is only possible from the MGMT zone or through the VPN with MFA (QNRCS PR.GA-3, Basic level: privileged accounts).

## 9.3 Firmware updates

- Firmware on all network devices (firewalls, routers, switches, APs) must be updated regularly, in line with the Vulnerability and Patch Management Policy.

- The deadlines for fixing vulnerabilities in network equipment are those of the Vulnerability and Patch Management Policy (section 4). Internet-facing equipment (firewall, VPN, router) takes priority: if a vulnerability is being actively exploited and there is no fix, the exposed service is switched off or isolated immediately.

- Before applying critical updates, a backup of the device configuration must be made.

- Devices no longer supported by the manufacturer (EOL) must be replaced or have documented compensating protective measures.

# 10. Monitoring and Event Logging

Network monitoring is essential to detect threats in good time and to have evidence in the event of an incident. Without event records (logs), incident response becomes practically impossible.

## 10.1 Logging requirements per component

| Component | Events to log | Minimum retention |
| --- | --- | --- |
| Firewall | Accepted and rejected connections; modified rules; administrator authentications. | [12 months] |
| VPN | Connections established and terminated; authentication failures; source IP addresses. | [12 months] |
| Managed switches | Configuration changes; port connections and disconnections. | [12 months] |
| Wi-Fi access points | Associations and disassociations; authentication failures; configuration changes. | [12 months] |
| DNS/DHCP servers | IP address assignments with MAC address and time. | [12 months] |
| Management equipment (MGMT) | All administrative access; configuration changes. | [12 months] |

Regulatory reference: RJC, Article 27(1)(a) and (e); QNRCS PR.SP-2 (Basic level: logs collected, stored, protected and reviewed regularly, with retention set according to risk) and DE.MC-1 (Basic level: monitoring of network traffic and protection against unauthorised and malicious activity). ISO/IEC 27002:2022, controls 8.15 (logging) and 8.16 (monitoring activities).

## 10.2 Protection and integrity of logs

- Logs must be stored on a system separate from the equipment that generates them (e.g. a central logging server or a cloud service).

- Access to logs must be restricted to the IT Manager and authorised personnel.

- Critical alerts (intrusion attempts, out-of-hours access, anomalous traffic) must trigger immediate notifications to the IT Manager.

- Logs must not be modifiable by the systems that generated them.

- Equipment synchronises its clock via NTP (Network Time Protocol) with a reliable time source, so that logs from different equipment can be compared (mandatory from the Substantial level, with synchronisation to an official clock source; QNRCS PR.SP-2).

- At the High level the following are added: an alert when log collection fails, periodic review of log storage capacity, and logs of critical systems kept on a standalone system (QNRCS PR.SP-2).

## 10.3 Alerts and review

- The IT Manager reviews the logs and alerts of the firewall, the VPN and the management equipment at least once a month and records the review in the log in section 14 (date, what was analysed, actions).

- Any alert of an intrusion attempt, unauthorised access or anomalous traffic must be investigated and, if confirmed, handled in accordance with the CIRP.

- The perimeter firewall has the protection functions it offers against malicious activity enabled and up to date (e.g. intrusion prevention (IPS), blocking of malicious destinations) (QNRCS DE.MC-1, Basic level). From the Substantial level, unauthorised local and remote communications to critical systems are monitored and blocked; at the High level, monitoring of critical systems is continuous and triggers automatic responses. In an SME, these levels are usually achieved through a contracted monitoring service.

- If the network becomes unavailable or compromised during an incident, the team uses the alternative means of communication defined in the CIRP (RJC, Article 27(1)(i): secured emergency communication systems).

# 11. Connections to Cloud Services

Adopting cloud services extends the traditional network perimeter and requires specific controls to ensure the security of connections and of the data transmitted:

- Connections between the corporate network and cloud environments (IaaS, PaaS, SaaS) must be encrypted (TLS 1.2 as a minimum, TLS 1.3 recommended).

- Access to critical cloud services must be subject to MFA and, preferably, restricted to corporate IP addresses or made through the VPN.

- Administrative access to cloud platforms must be managed through dedicated accounts with least privilege.

- Activity logs of cloud platforms must be enabled and retained in accordance with the requirements of this policy.

- Firewall rules and security groups in the cloud must follow the same 'deny by default' principle applied to the local network.

- Critical cloud service providers are assessed under the Supply Chain Risk Management Policy (annually or whenever relevant; QNRCS GR.CA-7).

# 12. Roles and Responsibilities

| Role | Responsibilities |
| --- | --- |
| Management Body | Approve this policy and the necessary resources and oversee its implementation (RJC, Article 25(1)(a) and (b)). Be informed of significant network incidents and serious breaches of this policy. |
| Cybersecurity Officer | Propose this policy and its revisions to the Management Body (RJC, Article 31). Approve exceptions to this policy (permissive rules, access without VPN or ZTNA). Follow up the results of the periodic reviews and ensure that external providers comply with this policy and their contracts. |
| IT Manager (internal or external provider) | Implement and maintain the secure network architecture. Manage firewalls, VLANs, VPN and Wi-Fi. Review the firewall rules and the remote access list every six months and the logs monthly. Document the network and keep the network equipment inventory up to date. Carry out or supervise configuration changes. |
| Staff | All personnel, including temporary staff and interns, and third parties with network access. Do not connect unauthorised devices to the network or create Wi-Fi access points. Report immediately to the IT Manager any anomaly or suspicious behaviour on the network. |

Regulatory reference: RJC, Article 25(1)(a) and (b) (the Management Body approves the cybersecurity risk-management measures, including this policy, and oversees their implementation), and Article 31 (Cybersecurity Officer).

# 13. Compliance and Review

## 13.1 Non-compliance

Non-compliance with this policy may result in restriction or revocation of network access, disciplinary proceedings under the Portuguese Labour Code and, in serious cases, civil or criminal liability. Connecting unauthorised devices to the corporate network, creating unauthorised access or changing network configurations without approval are considered serious breaches.

## 13.2 Policy review

This policy is reviewed annually, or whenever there are significant changes to the network architecture, the technology used or the legal framework, or after relevant security incidents, and is approved by the Management Body. The review can be scheduled as a recurring task in the NIS2PME platform. The next review is scheduled for [date].

# 14. Periodic Reviews and Checklist

The IT Manager uses this checklist in the six-monthly network security review and after significant changes to the infrastructure. Each review (six-monthly, or monthly for logs) is recorded in the log at the end of this section.

| Physical Security and Equipment |  |
| --- | --- |
| Critical network equipment in racks with restricted physical access. | [ ] |
| Unused switch ports are disabled. | [ ] |
| Network cables organised, protected and labelled. | [ ] |
| All equipment has up-to-date firmware; no EOL equipment without a replacement plan. | [ ] |
| Default administration credentials changed on all equipment. | [ ] |

| Segmentation and Firewall |  |
| --- | --- |
| Network architecture with VLANs segmented as defined in this policy. | [ ] |
| All connections between zones go through a firewall with explicit rules. | [ ] |
| 'Deny by default' principle applied on all firewalls. | [ ] |
| Firewall rules documented with justification and approval date. | [ ] |
| Firewall rules reviewed every six months; obsolete rules removed. | [ ] |
| Guest VLAN (GUEST) without access to internal networks. | [ ] |
| IoT VLAN isolated from user and server networks. | [ ] |

| VPN and Remote Access |  |
| --- | --- |
| VPN with a modern protocol (IKEv2/IPsec, OpenVPN or WireGuard). | [ ] |
| MFA mandatory for all VPN users. | [ ] |
| Split tunnelling disabled by default. | [ ] |
| VPN access logs kept for at least [12 months]. | [ ] |
| PPTP and L2TP without IPsec blocked on the firewall. | [ ] |
| Third-party VPN accounts with minimum access and automatic expiry. | [ ] |
| List of authorised remote access reviewed; unnecessary access removed. | [ ] |

| Wi-Fi |  |
| --- | --- |
| Internal Wi-Fi networks use WPA3 or WPA2-Enterprise (802.1X), except for a recorded temporary exception. | [ ] |
| Guest network isolated, with Internet access only and client isolation enabled. | [ ] |
| No WEP, WPA+TKIP or open networks in place. | [ ] |
| AP firmware up to date. | [ ] |
| Guest network password changed quarterly. | [ ] |
| No unauthorised AP (rogue AP) detected on the network. | [ ] |

| Monitoring and Documentation |  |
| --- | --- |
| Firewall and VPN logs centralised and kept for at least [12 months]. | [ ] |
| Critical security alerts notified to the IT Manager. | [ ] |
| Monthly review of logs and alerts by the IT Manager, recorded in the review log. | [ ] |
| Network diagram up to date (physical copy in a secure place). | [ ] |
| Network device inventory up to date. | [ ] |
| IP addressing table documented. | [ ] |
| Annual review of this policy completed and documented. | [ ] |

Review log

| Date | Review | Outcome and actions | Person responsible |
| --- | --- | --- | --- |
| [dd/mm/yyyy] | Firewall rules, remote access and Wi-Fi (six-monthly) | [e.g. 3 rules removed] | [name] |
| [dd/mm/yyyy] | Logs and alerts (monthly) | [summary] | [name] |
| [dd/mm/yyyy] | [type of review] | [summary] | [name] |

# 15. QNRCS Controls Covered

QNRCS controls (Regulation No. 756/2026, Annex III) for which this policy provides evidence. The level shown is the one the document meets as it stands. The technical evidence an auditor asks for (firewall, VPN and Wi-Fi configuration, logs) is in the equipment itself; this policy sets the rules and the records to keep.

| Code | Level | Where in this document |
| --- | --- | --- |
| PR.RI-1 | Basic; Substantial with the network diagram (9.1) | 5, 6, 8, 9.1 and 14 |
| PR.SP-1 | Basic | 6.2, 9.1 (recorded baseline configuration) and 9.2 |
| PR.SP-2 | Basic; Substantial with time synchronisation (10.2) | 10 and review log (14) |
| DE.MC-1 | Basic | 10.3 |
| PR.GA-3 | Basic, in part: remote access and network administration (the other accounts are in the Identity, Authentication and Access Control Policy) | 7 and 9.2 |
| PR.GA-7 | Basic; Substantial with remote maintenance (7) | 7 and 7.2 (remote access register) |
| PR.SD-2 | Basic, in part: data in transit on the network (VPN, TLS, SSH) | 7, 9.2 and 11 |
| PR.SP-3 | Basic, in part: network and web filtering (blocking of unauthorised software and e-mail filtering are missing) | 6.2 |
| PR.GA-6 | Basic, in part: locations of network equipment | 4 |
| ID.GA-3 | Substantial (diagram and internal flows); High with the approved external flows | 9.1 |
| PR.RI-2 | Substantial | 5.2 |

At the High level the following are also added (QNRCS PR.RI-1 and PR.SP-1): authenticated proxy servers between critical systems and external networks; fault tolerance in protection equipment; control of the data flows of critical systems; and configuration of critical systems with minimum functionality (unnecessary protocols, ports and services disabled). The “deny by default, allow by exception” principle is already applied by this policy at all levels.

# 16. Approval

| Prepared by | Reviewed by | Approved by the Management Body |
| --- | --- | --- |
| [Name] [Role] [Date] | [Name] [Role] [Date] | [Name] [Position] [Date] [Signature] |

Next review: [dd/mm/yyyy]. Approval under RJC, Article 25(1)(a).